Name:

Regents Earth Science

River Discharge Lab

Lab Day:		 Lab Period:

Lab Instructor:

INTRODUCTION:  Two important factors in the ability of a river to erode (transport sediment) are its CAPACITY and its COMPETENCE.  Look up those two terms in your text and make sure you understand them.

The capacity of a river depends on several variables.

One of them is the velocity of the river.  The faster the water is flowing, the more and larger the particles it can pick up and carry.

Equally important, however, is the amount of water in a river.  It should be obvious to you that, all other things being equal, the more water there is, the more material can be moved.

But it isn’t very worthwhile to talk about the “volume” of water in a river…a river with lots of water in it that flows very slowly may carry less sediment than a small mountain stream that flows very rapidly.

What really matters is how quickly the volume of water moves through the river system - or the volume of water that passes through the river system in certain amount of time.

Hydrologists (geoscientists that study rivers) call the measurement of the volume of water passing a point on the river in a specific amount of time the DISCHARGE of the  river.  The data needed to calculate the discharge of a stream is collected at a gauging station, and includes depth soundings and velocity measurements.

List 2 metric units of volume  ________________     _______________

List 2 units that measure time ________________     _______________

Use combinations of the units you listed above to list 4 possible units of DISCHARGE.

________________     _______________      _______________     _______________

In this lab, you will determine the discharge of the same stream from data collected in April and data collected in August.

By the end of the lab, you should be able to:

1. Draw the profile of a stream bed, given the depth data from a gauging station

2. Determine the x-sectional area of a stream bed a gauging station.

3. Determine the velocity of a stream

4. Calculate the discharge of a stream

5. Demonstrate your understanding of the term DISCHARGE by explaining what it is, how it is measured, and how it might change over time.

PROCEDURE:  Read every word of every sentence carefully before beginning.  Make sure you understand what you are reading.  Then go back over what you’ve read.  Learn from the descriptions and perform the tasks described in ITALICIZED CAPITAL LETTERS.

PART 1:  THE GAUGING STATION AND X-SECTIONAL PROFILE

A gauging station can be established at any convenient point along a river.  A simple station can be set up by stretching a string, supported by two stakes, across the stream as shown in Figure 1.  The string should be as close to the high water level of the river as possible.

FIGURE 1

“Soundings”, or depth measurements, can be made at regular intervals with a meter stick, as shown in Figure 2.  In the example shown here, soundings are being made at 10 cm. intervals.  The researcher must be careful to keep the meter stick vertical and measure the depth of the water accurately.

FIGURE 2

The sounding data collected from the stream used in this study are summarized in Tables 1 and 2 below:

TABLE 1	TABLE 2

Stream in AUGUST	Stream in APRIL

Distance from Shore	Depth	Distance from Shore	Depth

(cm)	(cm)	(cm)	(cm)

0	0	0	0

10	3	10	3

20	7	20	8

30	13	30	12

40	18	40	12

50	18	50	21

60	24	60	22

70	26	70	25

80	24	80	32

90	29	90	35

100	31	100	39

110	26	110	44

120	27	120	47

130	23	130	44

140	20	140	42

150	17	150	35

160	10	160	29

170	0	170	25

180	20

190	15

200	8

210	8

220	0

DRAW A GRAPH OF THE STREAM DEPTH DATA.  Since the depth of the stream varies with distance from the shore, the independent variable is:

____________________________

and should be labeled on the X-axis

LABEL THE DEPENDANT VARIABLE (DEPTH) ON THE Y-AXIS.  Your graph will be most useful to you if 0 is at the top of the y-axis and the depth values increase downward from 0.

Once your graphs are neatly drawn and labeled, DETERMINE THE X-SECTIONAL AREA OF THE STREAM IN AUGUST AND AGAIN IN APRIL.  There are several ways to do this, and you should even be able to write a formula to do it.  THINK HARD, BUCKAROOS! AND ENTER YOUR CALCULATED AREAS BELOW.  Remember, no calculation is complete without units.

X-SECT. AREA	X-SECT. AREA

of August Stream	of April Stream

_______________	_______________

PART 2:  MEASURING VELOCITY

The velocity of the stream was measured by timing a wooden block as it was carried by the water for distance of 500 cm (5 m).  Several measurements were made - the results appear in Table 3 and Table 4.

TABLE 3	TABLE 4

STREAM IN AUGUST	STREAM IN APRIL

TIME for wooden block	TIME for wooden block

to travel 500 cm	to travel 500 cm

(sec)	(sec)

43	23

47	27

50	29

39	24

51	30

50	18

USE THE DATA IN TABLES 3 AND 4 TO DETERMINE THE AVERAGE VELOCITY OF THE STREAM AT THE TIME THE MEASUREMENTS WERE MADE, AND ENTER THAT INFORMATION BELOW:

VELOCITY	VELOCITY

of stream in	of stream in

AUGUST	APRIL

_______________	_______________

PART 3: DETERMINING DISCHARGE

DETERMINE THE DISCHARGE OF THE EACH STREAM MULTIPLYING THE X-SECTIONAL AREA BY THE VELOCITY, AND ENTER THAT INFORMATION HERE.

DISCHARGE OF	DISCHARGE OF

Stream in AUGUST	Stream in APRIL

__________________	__________________

WRITE A FORMULA FOR THE DISCHARGE OF A RIVER:

_________________________________________